REAL COMMAND OUTPUTS

*IDN? - HELP? - SYST:STAT?

*IDN?

Jackson Labs, LN Rb GPSDO (PRE), Firmware Rev 1.17

HELP?

*IDN?

SERVo?

SERVo:DACGain    <float>[0.001, 10000]

:MODE       <SLOW|MEDium|FAST|AUTO>

:STATE?

:EFCScale   <float>[0.0, 500.0]

:EFCDamping <int>  [2, 4000]

:FILTerlength <int>  [2, 20]

:TEMPCOmpensation  <float> [-4000.0, 4000.0]

:AGINGcompensation <float> [-10.0, 10.0]

:PHASECOrrection   <float> [-500.0, 500.0]

:1PPSoffset        <int> ns

:QUIet   <ON|OFF>

:TRACe   <int>  [0,255]

:TRACe:PORT <RS232|USB>

:LOOP    <ON|OFF>

:COARSedac [0,255]

:SELect <CSAC:FILTer>

SYNChronization?

SYNChronization:SOURce:MODE <GPS|EXTernal|AUTO>

SYNChronization:SOURce:STATE?

SYNChronization:HOLDover:DURation?

:HOLDover:STATe?

:HOLDover:INITiate

:HOLDover:RECovery:INITiate

:TINTerval?

:TINTerval:CSAC?

:TINTerval:FILTer?

:TINTerval:THReshold <int> [50,2000]

:IMMEdiate

:FEEstimate?

:LOCKed?

:OUTput:FILTer  <ON|OFF>

:OUTput:1PPS:RESET  <ON|OFF>

:OUTput:1PPS:DOMAIN <CSAC|FILTer>

:HEAlth?

MAC?

MAC:RS232?

:STeer?

:STATus?

:MODE?

:DDScenter?

:TECcontrol?

:TCXO?

:SIGnal?

:HEATpackage?

:TEMPerature?

:SN?

:FWrev?

:LIFEtime?

:STeer:LATch  ONCE

:PASSthrough <csac_cmd>

DIAGnostic?

DIAGnostic:ROSCillator:EFControl:RELative?

RELative:CSAC?

RELative:FILTer?

ABSolute?

ABSolute:CSAC?

ABSolute:FILTer?

:LIFetime:COUNt?

GPS?

GPS:RESET   ONCE

GPS:REFerence:ADELay  <float> <s|ns> [-32767ns,32767ns]

GPS:REFerence:PULse:SAWtooth?

GPS:TMODe   <ON|OFF|RSTSURV>

GPS:SURVey  ONCE

GPS:SURVey:STATus?

:DURation  <int in sec>

:VARiance  <int>

GPS:HOLD:POSition <int,int,int>

GPS:DYNAMic:MODE  <int>  [0,8]

GPS:DYNAMic:STATe?

GPS:PORT     <RS232|USB>

GPS:GPRMC    <int>  [0,255]

GPS:GPGGA    <int>  [0,255]

GPS:GGASTat  <int>  [0,255]

GPS:XYZSPeed <int>  [0,255]

GPS:GPZDA    <int>  [0,255]

GPS:PASHR    <int>  [0,255]

GPS:GPGSV    <int>  [0,255]

GPS:SATellite:TRAcking:COUNT?

:VISible:COUNT?

GPS:POSition?

GPS:POSition:ECEF?

GPS:JAMlevel?

GPS:FWver?

GPS:INITial:DATE <yyyy,mm,dd>

:TIME <hour,min,sec>

YSTem:SELect  [GPS | SBAS | QZSS | GAL | BD ^ GLO]

GYRO?

GYRO:MODE        <ON|OFF>

GYRO:TRACE       <int>  [0,255]

GYRO:CALibrate   <float,float,float,float,float,float>

GYRO:CALibrate:COMPute

GYRO:CALibrate:RESET

GYRO:SENSitivity <float,float,float>

GYRO:GLOAD?

GYRO:PORT        <RS232|USB>

MEASure?

MEASure:TEMPerature?

:VOLTage?

:CURRent?

:POWersupply?

PTIMe?

PTIMe:DATE?

PTIMe:TIME?

PTIMe:TIME:STRing?

PTIMe:TINTerval?

PTIMe:OUTput <ON|OFF>

PTIMe:LEAPsecond?

PTIMe:LEAPsecond:PENDing?

:ACCumulated?

:DATE?

:DURation?

SYSTem:COMMunicate:SERial:ECHO   <ON|OFF>

:PROmpt <ON|OFF>

:BAUD   <9600 | 19200 | 38400 | 57600 | 115200>

:SERVo  <CSAC | FILTer>

SYSTem:COMMunicate:USB:BAUD      <9600 | 19200 | 38400 | 57600 | 115200>

:SERVo  <CSAC | FILTer>

SYSTem:STATus?

SYSTem:ID?

SYSTem:ID:SN?

SYSTem:ID:HWrev?

SYSTem:LCD:CONTrast <float> [0,1.0]

SYSTem:LCD:PAGE     <int> [0,9]

SYSTem:FACToryreset ONCE

HELP?

SYST:STAT?

LN Rb GPSDO (PRE)  Serial Number : 122301668    Hw version : 1.01.00

AQUISITION ................................................

Tracking:14        Not Tracking: 2

PRN  El  Az   SS   PRN  El  Az

1  87 250   23    19   4 329

2  65 159   31    308   1  40

4  30 220   30

8  10 194   10

17  25 303   16

28  34  78   38

31  34 115   47

32  21  47   41

40  38 142   32

41  18 116   41

307  49  53   33

326  60 120   30

329  68 183   24

333  63 320   19

LAT   N 40:47:13.689             LON   E 29:27:12.366

HGT   147.60 m (MSL)             UTC    7:06:46   2 Sep 2026

HEALTH MONITOR................................................

GPS Receiver Status: 3D Fix             GPSDO Status : Locked

1PPS SOURCE MODE  : GPS                 1PPS SOURCE STATE  : GPS

SYST:STAT?

LN Rb GPSDO (PRE)  Serial Number : 122301668    Hw version : 1.01.00

AQUISITION ................................................

Tracking:14        Not Tracking: 6

PRN  El  Az   SS   PRN  El  Az

1  83 178   30     8   5 192

2  58 161   33     9   4 228

3  58 312   19    19   8 325

4  36 223   25    312  15 312

17  27 296   15    313   5 130

28  33  69   40    319  26 213

31  38 107   46

32  15  48   31

40  38 142   32

41  18 116   41

307  45  50   41

326  55 127   37

329  74 181   26

333  69 319   24

LAT   N 40:47:13.813             LON   E 29:27:13.291

HGT   192.10 m (MSL)             UTC    7:21:20   2 Sep 2026

HEALTH MONITOR................................................

GPS Receiver Status: 3D Fix             GPSDO Status : Locked

1PPS SOURCE MODE  : GPS                 1PPS SOURCE STATE  : GPS

GPS?

GPS?

GNSS: GPS SBAS GAL

ANTENNA DELAY:2.5e-08

PULSE SAWTOOTH:-6.0

TRACKED SATS :13

VISIBLE SATS :21

ACTUAL POSITION:

N,4047.2368

E,2927.2267

190.00 m

0.10 Knots

0.00 Degrees

GPS Receiver Status: 3D Fix

DYNAMIC MODE:AUTOMATIC(8)

DYNAMIC STATE:STATIONARY(1)

SURVEY MIN DURATION:3600

SURVEY VARIANCE LIMIT:150000

SURVEY STATUS:ACTIVE

Duration : 2580, ECEF 421116328,237806312,414469764, 3D variance 274822336

TIMING MODE:RSTSURV

HOLD POSITION:-267998828,-430158341,385929826

JAMMING LEVEL:5

FIRMWARE VERSION: 3.01

GPS?

GNSS: GPS SBAS GAL

ANTENNA DELAY:2.5e-08

PULSE SAWTOOTH:-3.1

TRACKED SATS :13

VISIBLE SATS :20

ACTUAL POSITION:

N,4047.2281

E,2927.2189

182.90 m

0.09 Knots

0.00 Degrees

GPS Receiver Status: 3D Fix

DYNAMIC MODE:AUTOMATIC(8)

DYNAMIC STATE:STATIONARY(1)

SURVEY MIN DURATION:3600

SURVEY VARIANCE LIMIT:150000

SURVEY STATUS:ACTIVE

Duration : 1270, ECEF 421116113,237805586,414468927, 3D variance 266913968

TIMING MODE:RSTSURV

HOLD POSITION:-267998828,-430158341,385929826

JAMMING LEVEL:8

FIRMWARE VERSION: 3.01

GPS?

GNSS: GPS SBAS GAL

ANTENNA DELAY:2.5e-08

PULSE SAWTOOTH:-5.4

TRACKED SATS :14

VISIBLE SATS :20

ACTUAL POSITION:

N,4047.2319

E,2927.2278

187.30 m

0.03 Knots

0.00 Degrees

GPS Receiver Status: 3D Fix

DYNAMIC MODE:AUTOMATIC(8)

DYNAMIC STATE:STATIONARY(1)

SURVEY MIN DURATION:3600

SURVEY VARIANCE LIMIT:150000

SURVEY STATUS:ACTIVE

Duration : 2798, ECEF 421116317,237806519,414469904, 3D variance 276520608

TIMING MODE:RSTSURV

HOLD POSITION:-267998828,-430158341,385929826

JAMMING LEVEL:11

FIRMWARE VERSION: 3.01

GPS?

(BURDA ANTENDEN SİNYAL ALAMIYO)

GNSS: GPS SBAS GAL

ANTENNA DELAY:2.5e-08

PULSE SAWTOOTH:0.0

TRACKED SATS :0

VISIBLE SATS :0

ACTUAL POSITION:

N,   0.0000

E,   0.0000

0.00 m

0.00 Knots

0.00 Degrees

GPS Receiver Status: No Fix

DYNAMIC MODE:AUTOMATIC(8)

DYNAMIC STATE:STATIONARY(1)

SURVEY MIN DURATION:3600

SURVEY VARIANCE LIMIT:150000

SURVEY STATUS:ACTIVE

Duration : 14, ECEF 0,0,0, 3D variance 4294967295

TIMING MODE:RSTSURV

HOLD POSITION:-267998828,-430158341,385929826

JAMMING LEVEL:4

FIRMWARE VERSION: 3.01

GPS:SAT:TRAC:COUN?

20

GPS:SAT:VIS:COUN?

19

ŞUAN BİDAHA DENEDİM COMMAND ERROR VERİYO ???? SIKINTI

Nasıl böyle oluyo, niye fark var ???

PTIME?

PTIME?

DATE :2026,9,2

TIME :7:15:14

TINTerval :9.063E-09

OUTput :0

LEAPSECOND :18

PTIME?

DATE :2026,9,2

TIME :7:44:23

TINTerval :2.672E-09

OUTput :0

LEAPSECOND :18

PTIME:DATE?

2026,9,2

PTIME:TIME?

7,46,13

PTIME:TIME:STRING?

7:46:33

SYNC?

SYNC?

1PPS SOURCE MODE  : GPS

1PPS SOURCE STATE : GPS

1PPS on RESET : OFF

1PPS DOMAIN : CSAC

1PPS LOCK STATUS  : 1

HOLDOVER STATE: NONE

LAST HOLDOVER DURATION : 191,0

FREQ ERROR ESTIMATE : 1.31E-11

TIME INTERVAL DIFFERENCE : 1.133E-08

TIME INTERVAL THRESHOLD : 220

PHASE NOISE FILTER : ON

HEALTH STATUS : 0x0

SYNC?

1PPS SOURCE MODE  : GPS

1PPS SOURCE STATE : GPS

1PPS on RESET : OFF

1PPS DOMAIN : CSAC

1PPS LOCK STATUS  : 1

HOLDOVER STATE: NONE

LAST HOLDOVER DURATION : 191,0

FREQ ERROR ESTIMATE : 2.49E-11

TIME INTERVAL DIFFERENCE : -7.873E-09

TIME INTERVAL THRESHOLD : 220

PHASE NOISE FILTER : ON

HEALTH STATUS : 0x0

SYNC?

1PPS SOURCE MODE  : GPS

1PPS SOURCE STATE : GPS

1PPS on RESET : OFF

1PPS DOMAIN : CSAC

1PPS LOCK STATUS  : 1

HOLDOVER STATE: NONE

LAST HOLDOVER DURATION : 191,0

FREQ ERROR ESTIMATE : 1.59E-11

TIME INTERVAL DIFFERENCE : -1.179E-08

TIME INTERVAL THRESHOLD : 220

PHASE NOISE FILTER : ON

HEALTH STATUS : 0x0

SYNC:LOCKED?

1

SYNC:TINT?

6.763E-08

SYNC:HEALTH?

0x1000

SYNC:HOLD:DUR?

150,0

SYNC:HOLD:INIT

SYNC:HOLD:REC:INIT

SYNC:SOUR:MODE <m>

bunlar çok lazım değilmiş

DIAG? - MEAS? - CSAC?

DIAG?

EFControl Relative: -0.410000%

EFControl Absolute: -82.000000

Lifetime : +0

DIAG?

EFControl Relative: -0.440000%

EFControl Absolute: -88.000000

Lifetime : +0

diag?

EFControl Relative: -0.640000%

EFControl Absolute: -128.000000

Lifetime : +0

DIAG?

EFControl Relative: -0.105000%

EFControl Absolute: -21.000000

Lifetime : +0

MEAS?

PCB Temperature: 46.5688

CSAC Temperature: 47.83

TCXO Voltage: 1.672

Power Supply Voltage: 11.71

MEAS?

PCB Temperature: 52.7762

CSAC Temperature: 54.10

TCXO Voltage: 1.659

Power Supply Voltage: 11.74

MEAS?

PCB Temperature: 52.8652

CSAC Temperature: 54.17

TCXO Voltage: 1.659

Power Supply Voltage: 11.73

MEAS:TEMP?

51.1479

MEAS:CURR?

51.3210

MEAS:VOLT?

1.662

CSAC?

RS232: OK

STEER: -71.000

STATUS: 0

MODE: 0x0000

TEC CONTROL: 56.22

DDS CENTER: 0.00

TCXO VOLTAGE: 1.658

DC SIGNAL LEVEL: 1.00

HEAT PACKAGE: 2730.00

TEMPERATURE: 54.36

SN: 2209MX04906

FIRMWARE REV: V1.0.23

LIFETIME: 11247

CSAC?

PCB Temperature: 49.7419

CSAC Temperature: 51.13

TCXO Voltage: 1.665

Power Supply Voltage: 11.72

Command Error

RS232: OK

STEER: -15.000

STATUS: 0

MODE: 0x0000

TEC CONTROL: 56.24

DDS CENTER: 0.00

TCXO VOLTAGE: 1.665

DC SIGNAL LEVEL: 1.00

HEAT PACKAGE: 3000.00

TEMPERATURE: 51.39

SN: 2209MX04906

FIRMWARE REV: V1.0.23

LIFETIME: 11248

CSAC:STATUS?

0

CSAC:TEMP?

54.65

CSAC:SN?

2209MX04906

EXTRA COMMANDS (NOT İMPLEMENTED YET, MAYBE WE CAN I WİLL ASK THIS)

MAC?

RS232: OK

STEER: -97.000

STATUS: 0

MODE: 0x0000

TEC CONTROL: 56.29

DDS CENTER: 0.00

TCXO VOLTAGE: 1.682

DC SIGNAL LEVEL: 1.00

HEAT PACKAGE: 3655.00

TEMPERATURE: 43.74

SN: 2209MX04906

FIRMWARE REV: V1.0.23

LIFETIME: 11248

GYRO?

MODE : 0

TRACE: 0

CALIBRATION : Offset: 0.000, 0.000, 0.000, Gain: 1.0000, 1.0000, 1.0000

G-SENSITIVITY : X 0.000 mHz/g, Y 0.000 mHz/g, Z: 0.000 mHz/g

GLOAD : -0.067,-0.045,-1.063

PORT : RS232

GPS:POSITION?

N,4047.2327

E,2927.2257

194.70 m

0.07 Knots

0.00 Degrees

SERV?

SERVO: CSAC

LOOP: 1

DAC GAIN: 2.000

EFC SCALE : 0.50

PHASE CORRECTION : 1.500000

EFC DAMPING: 10

FILTER LENGTH: 20

TEMPERATURE COMPENSATION : 0

AGING COMPENSATION : -0.000547502

1PPS OFFSET : 0.000 ns

TRACE PORT : RS232

TRACE : 0

FASTLOCK : 1

FASTLOCK PERIOD  : 1800